ESP32 Automatic Weather Station

Firmware Redesign: Time-Based Sampling with Relay Power Gating

Technical Documentation and Handover Walkthrough

| Item | Detail |
| --- | --- |
| Document date | 14 August 2026 |
| Scope of change | src/main.cpp only. No other file modified. |
| Branch | fix/time-based-scheduling |
| Target | ESP32-WROOM-32, PlatformIO board esp32dev, Arduino framework |
| Toolchain verified | arduino-esp32 2.0.17, ESP-IDF 4.4.7 |
| Build status | Compiles clean, zero warnings. Flash 23.6 percent, RAM 6.8 percent. |
| Hardware status | Not yet verified on hardware. See section 9. |

# Contents

1. Summary	3

1.1 What changed	3

1.2 Verification status	3

2. Background and design rationale	3

2.1 The existing firmware	3

2.2 The design document and where it diverges	3

2.3 Why simple polling was rejected	4

3. Architecture	4

3.1 Program structure	4

3.2 Wind and rain: hardware pulse counting	5

3.2.1 Free running counters and delta reads	5

3.2.2 Disabling limit events	5

3.2.3 Pull up configuration	5

3.2.4 Rain: capture in hardware, debounce in software	5

3.2.5 Wind: accumulation and gust sub sampling	5

3.3 Relay control and the sensor rail	6

3.3.1 Re-initialisation on every power up	6

3.3.2 Bus release and back powering	6

3.3.3 Warmup	6

3.3.4 Relay wear protection	6

3.4 The scheduler	6

4. Operation	7

4.1 Timeline of one sample cycle	7

4.2 Power	7

4.3 Relay service life	8

4.4 Serial console reference	8

5. Diagnostics and error handling	9

6. Bring up checklist	9

7. Open items and decisions required	10

7.1 Wind calibration constant, action required	10

7.2 Sleep modes are not used	10

7.3 Interval persistence	10

7.4 Interval ceiling	10

7.5 Questions still outstanding for the team	10

8. Configuration reference	11

9. Limitations and verification status	11

# 1. Summary

The weather station firmware has been rewritten to move sampling from an interrupt driven design to a time based design, and to use the relay as a real power gate rather than leaving it energised permanently. The pin map is unchanged. No hardware modification is required.

## 1.1 What changed

- No interrupts. There is no attachInterrupt call and no interrupt service routine anywhere in the file. Wind and rain pulses are counted by the ESP32 PCNT (Pulse Counter) peripheral, which tallies edges in dedicated hardware, independently of the processor. The firmware reads a register on a schedule.

- No blocking delays. The main loop never calls delay(). All timing is a non blocking state machine driven by overflow safe millis() arithmetic.

- The relay is actually used. The sensor rail is energised only for the approximately 2.3 seconds needed to warm up and read the gated sensors, then switched off. At the default 300 second interval that is a 0.77 percent duty cycle. The previous firmware closed the relay once at start up and never opened it.

- Wind and rain stay powered. Both are passive reed switches and draw effectively no current, so gating them would save nothing and would lose data. They are counted continuously, including while the sensor rail is off.

- Diagnostics added. Per sensor health reporting, real time clock validation, a dead battery warning for the clock module, and a failure counter for the temperature and humidity sensor. Several failures that were previously silent are now visible.

## 1.2 Verification status

The firmware compiles cleanly with zero warnings and links successfully. The three I2C libraries were inspected directly to confirm that repeated initialisation, which now happens on every sample cycle, does not leak memory. See section 9 for what has not been verified.

# 2. Background and design rationale

## 2.1 The existing firmware

The previous version was a single file bench sketch. The relay on GPIO 13 was switched on during start up and left on for the life of the program. Wind and rain used hardware interrupts with a software debounce on rain only. The main loop read every sensor and printed the result, then blocked for two seconds.

## 2.2 The design document and where it diverges

The design document describes a more advanced system: light sleep as the idle state, secure digital card logging with an archive file and an upload queue, wireless upload to three separate cloud channels, long range radio, a configuration portal, and additional sensors that are not present in the current code.

Two of its core decisions are the opposite of the direction now being taken. The document specifies no relay and no power cut, with the board powered continuously, and it specifies that wind and rain remain interrupt driven at all times. The new hardware direction supersedes the document on both points. The team should confirm this is deliberate.

The document also contains a latent timing defect worth fixing there as well. Its pseudocode compares millis() >= nextCycleTime. The millisecond counter wraps to zero after approximately 49.7 days. At that point the counter becomes small while the stored deadline is still large, the comparison never becomes true, and the station stops sampling until the counter catches up roughly 49 days later. The overflow safe form, used throughout the new firmware, is millis() - lastRun >= interval.

## 2.3 Why simple polling was rejected

The literal request was to replace interrupts with timed polling of the sensor pins. That approach was not taken, for reasons that affect data integrity rather than style.

- A rain gauge tip closes the contact for only a few milliseconds. Catching that by polling a pin requires a loop running roughly every two milliseconds, which is 500 processor wake ups per second. That consumes more power than the interrupts it was meant to replace, because the processor can never idle.

- It still loses pulses. Any time the loop takes longer than one pulse width, that pulse is gone. A single temperature and humidity sensor read blocks for about 25 milliseconds on its own.

- Lost rain tips cannot be recovered. Rainfall is cumulative, so every dropped tip is 0.2 millimetres permanently missing from the record, and the error only ever grows in one direction. A season of data that reads 30 percent low is corrupt in a way that cannot be detected afterwards. It simply looks as though it rained less.

The hardware pulse counter satisfies the actual requirement, which was time based operation without interrupts, and loses nothing. The processor is not involved in counting at all.

# 3. Architecture

## 3.1 Program structure

The main loop is three non blocking calls:

void loop() {

serviceSerial();        // console

servicePulseCounters(); // wind accumulate + gust peak + rain debounce

serviceCycle();         // relay / warmup / read state machine

}

There is exactly one delay() call in the entire file, in start up, for serial port enumeration.

## 3.2 Wind and rain: hardware pulse counting

Each reed switch is assigned its own pulse counter unit: wind to unit 0, rain to unit 1. Both are configured to count falling edges only, which matches the previous interrupt configuration exactly.

### 3.2.1 Free running counters and delta reads

The counters are never cleared after start up. Each read subtracts the previous value using unsigned 16 bit arithmetic. This has two benefits. It wraps correctly at the counter boundary, and it removes the read then clear race in which a pulse arriving between those two operations would be lost.

### 3.2.2 Disabling limit events

This is the subtle configuration detail. The pulse counter hardware zeroes its counter whenever an enabled limit event fires. With a limit value of zero, that would clear the counter continuously and nothing would ever be counted. The setup routine therefore explicitly disables the high limit, low limit, both threshold events and the zero event, rather than relying on the reset state of those enable bits.

### 3.2.3 Pull up configuration

The unit configuration call claims the pin but does not set its pull mode, and a reed switch only ever pulls the line down. Without an explicit gpio_set_pull_mode(..., GPIO_PULLUP_ONLY) the input would float and produce spurious counts.

### 3.2.4 Rain: capture in hardware, debounce in software

An important correction to an earlier assessment: the pulse counter has a hardware glitch filter, but it is limited to 1023 peripheral clock cycles, which is 12.8 microseconds at 80 megahertz. That removes electrical noise and interference spikes. It does not remove mechanical contact bounce, which lasts for milliseconds. A software debounce is still required.

The division of labour is the point:

serviceRainDebounce()  - every 20 ms

delta = pcntTakeDelta(rain)

if delta > 0 and (now - lastAcceptedTip) >= 100 ms:

accept exactly ONE tip

discard the rest of delta as bounce

The hardware guarantees the tip was registered no matter how brief it was. The polling loop only decides how many of those registered edges were real. Polling the pin directly would miss the tip entirely. Polling a counter cannot.

Collapsing a burst into one tip is safe. Two genuine tips within 100 milliseconds would represent a rainfall rate of roughly 52,000 millimetres per hour.

### 3.2.5 Wind: accumulation and gust sub sampling

Every falling edge is a genuine revolution, so all deltas accumulate. Separately, the counter is sampled every three seconds, which is the standard meteorological gust window. Each short window is converted to a speed and the interval peak is retained.

This is a genuine trade off between power and data quality, so it is exposed as a single switch, GUST_TRACKING_ENABLED, at the top of the file. Gust cannot be recovered after the fact: 100 pulses spread over five minutes reads identically whether the wind was steady or completely calm with one violent burst.

## 3.3 Relay control and the sensor rail

### 3.3.1 Re-initialisation on every power up

Any device that loses power returns at factory defaults. All initialisation and configuration calls therefore live in sensorsInit(), which runs on every rail power up, rather than once at start up as before. This was the main structural change required by relay gating.

Because this now runs indefinitely, the three I2C libraries were checked for allocation leaks on repeated initialisation. The current sensor guards against re-allocation, and the analogue converter and clock libraries release the previous allocation before creating a new one. None of them leak. The current sensor library also rewrites its calibration register on each call, which is exactly what is needed after a power cut.

### 3.3.2 Bus release and back powering

This is the most commonly missed fault in power gated I2C designs. If the data and clock lines remain pulled high while a sensor supply pin is dead, current flows backwards through the sensor protection diodes into its supply rail. The device half wakes: it produces communication failures, corrupt readings, and occasional bus lock ups that only ever appear after a power cycle.

The rail shutdown routine therefore ends the I2C session and releases the data line, clock line and sensor data line to high impedance with no pull up, before the relay opens.

### 3.3.3 Warmup

The warmup period is 2200 milliseconds, sized for the temperature and humidity sensor, which is far slower to settle than any of the I2C devices. It is waited out without blocking, so pulse counting and the console continue to be serviced throughout.

### 3.3.4 Relay wear protection

Below a 30 second sample interval the rail is simply left energised. Cycling faster than that consumes relay service life for no meaningful power saving. The console reports a running count of relay operations so wear remains visible.

## 3.4 The scheduler

CYCLE_IDLE   -> interval elapsed (or manual trigger) -> rail on, go to WARMUP

CYCLE_WARMUP -> warmup elapsed -> init sensors, read, publish, drop rail

The rail power on routine stamps its timestamp only on a genuine off to on transition. If the rail has already been up for some time, the warmup state falls straight through. If the rail was forced on one second ago, the full warmup is still observed. The behaviour is correct in both directions with no special case.

The cadence does not drift. The scheduler advances by whole intervals rather than restarting from the current time, so the duration of each sample does not accumulate as error. If the station falls more than one interval behind, it resynchronises rather than firing a burst of catch up cycles.

The interval recorded against each reading is measured from one read to the next, not taken from the scheduler. Averages therefore stay correct even when a cycle runs late or is triggered manually partway through an interval.

Rainfall totals are computed from an integer tip count rather than by repeatedly adding to a floating point running total, which removes accumulated rounding error over months of operation.

# 4. Operation

## 4.1 Timeline of one sample cycle

At the default 300 second interval:

| Time | Event |
| --- | --- |
| 0.0 s | Scheduler fires. Relay closes, sensor rail comes up. |
| 0.0 to 2.2 s | Warmup. The loop keeps running: wind accumulating, gust sampled at three second boundaries, rain debounced every 20 milliseconds. |
| 2.2 s | Every device re-configured from scratch. |
| 2.2 to 2.3 s | Temperature and humidity read, four analogue conversions, two current sensors, clock timestamp. |
| 2.3 s | Bus released to high impedance, relay opens, rail down. |
| 2.3 s | Record printed to the serial console. |
| 2.3 to 300 s | Rail off. Wind and rain still counted in hardware, continuously. |

Resulting rail duty cycle: 0.77 percent.

## 4.2 Power

The following figures are approximate and depend on the specific relay fitted. They should be measured on the actual hardware.

| Consumer | Before | After |
| --- | --- | --- |
| Relay coil | Approximately 70 to 100 mA continuous, energised at start up and never released | Approximately 0.6 mA average at 0.77 percent duty |
| Gated sensors | Approximately 4 mA continuous | Approximately 0.03 mA average |
| Processor | Approximately 40 to 70 mA at 240 MHz | Approximately 25 to 30 mA at 80 MHz |

The single largest saving is not the sensor gating. It is no longer leaving the relay coil energised continuously. The previous firmware closed the relay during start up and never opened it, so the coil was drawing roughly 70 to 100 milliamps around the clock in order to switch a load of about 4 milliamps. Correcting that alone accounts for most of the improvement.

The processor clock reduction to 80 megahertz is applied before the serial port is opened, so the port divider is derived from the final clock configuration. The peripheral bus remains at 80 megahertz either way, so the pulse counter, I2C and serial port are unaffected. Set CPU_FREQ_MHZ to 240 to restore the original speed.

## 4.3 Relay service life

Relay ratings quote two different figures. The commonly cited 100,000 operations is the rating under electrical load. Switching only a few milliamps, contact erosion is negligible and the governing figure is the mechanical rating instead, typically around 10 million operations.

| Sample interval | Operations per year | Approximate service life |
| --- | --- | --- |
| 300 s (default) | Approximately 105,000 | Decades |
| 30 s (gating floor) | Approximately 1.05 million | Approximately 9 years |
| 5 s (if the floor were removed) | Approximately 6.3 million | Under 2 years |

The 30 second gating floor exists precisely to keep the station out of the bottom row. If the relay is later used to switch a heavier load, such as a heater, fan or radio module, these figures must be recalculated against the electrical rating rather than the mechanical one.

## 4.4 Serial console reference

Commands are typed followed by the Enter key. A 100 millisecond inactivity flush is also implemented, so terminals that do not transmit a newline still work.

| Command | Action |
| --- | --- |
| h | Show the command list |
| t | Status: rail mode, interval, cycle count, relay operations, rainfall total, pending wind pulses, sensor failure count, processor clock |
| s | Take a sample immediately |
| r | Reset the rainfall running total |
| a | Return the rail to automatic scheduled gating |
| 1 | Force the rail permanently on |
| 0 | Force the rail off. Sampling pauses, but wind and rain continue to be counted. |
| i<seconds> | Set the sample interval, for example i300 for five minutes, or i10 for bench testing |

For bench testing, i10 is the quickest way to see behaviour. Note that it will report relay gating as switched off, because 10 seconds is below the 30 second floor described in section 3.3.4.

# 5. Diagnostics and error handling

The following failures were previously silent and are now reported.

- Real time clock presence is now honoured. The initialisation result is checked and the returned date and time is validated, so a missing or unpowered clock reports its state explicitly instead of emitting a plausible looking but meaningless timestamp. The previous fallback string was unreachable code, because the formatting call always overwrote it.

- Clock backup battery is checked on every initialisation and warns clearly. Because the clock module sits on the gated rail, a flat or missing backup cell means timekeeping fails at every power cut. This makes the condition visible immediately rather than after a month of incorrectly stamped data.

- Temperature and humidity failures are counted and shown. The previous good value is still held when a reading fails, exactly as before, but a slowly degrading sensor is now visible instead of hidden.

- A per sensor health line is printed with every record, covering the analogue converter, both current sensors, the clock, and the temperature and humidity sensor.

- The timestamp buffer was increased from 20 bytes, which held the 19 character timestamp and its terminator with zero margin, to 24 bytes, and now uses a length limited formatting call.

# 6. Bring up checklist

Perform these steps in order on first flash.

- Flash the firmware and watch the start up banner. The station performs one immediate cycle rather than staying silent for a full interval.

- Confirm the health line reports every device as good. Any failure means that device did not return after the rail came up. Check the warmup period first.

- Watch for the clock backup battery warning.

- Spin the anemometer by hand, then type t and confirm the pending wind pulse count is increasing.

- Tip the rain bucket by hand several times. Confirm exactly one tip is counted per physical tip, not two or three. If over counting is observed, increase the rain debounce period.

- Type i10 and confirm records appear every ten seconds and the rail reports as permanently on.

- Type a, then i300, and leave the station running.

Steps 4 and 5 are the most important. They are the two behaviours that cannot be confirmed without hardware.

# 7. Open items and decisions required

## 7.1 Wind calibration constant, action required

The calibration constant was deliberately left at the existing value of 1.5 so that this rewrite does not silently alter the recorded dataset. However, the published specification for the Davis 6410 anemometer is:

V(mph) = 2.25 * P / T

If that is the anemometer fitted, then the correct constant is 2.25 and every wind reading is approximately 33 percent low. The previous metres per second constant of 0.67 was internally consistent with 1.5, so both figures were wrong by the same factor. This affects historical data as well as new readings.

The fix is a single line at the top of the file, WIND_MPH_PER_HZ. Someone should confirm the anemometer model and then make the change deliberately.

## 7.2 Sleep modes are not used

This is intentional. The pulse counter is clocked from the peripheral bus, which is gated during light sleep, so the counters would stop and pulses would be lost. Power saving in this design comes from the relay and the reduced processor clock instead.

Going further means moving pulse counting to the ultra low power coprocessor or to an external counter chip. That is a larger change and a hardware discussion, not a firmware adjustment.

## 7.3 Interval persistence

The interval command takes effect immediately but does not survive a reboot. Persisting it to non volatile storage, as the design document describes, is a small addition and requires no new library, but it was outside the agreed scope for this change.

## 7.4 Interval ceiling

The maximum interval is capped at 3600 seconds because the pulse counter register is 16 bit. At maximum wind speed the counter would saturate in roughly 24 minutes, so one hour has adequate margin but not unlimited headroom.

## 7.5 Questions still outstanding for the team

- Is the design document still the target for the full system, covering card logging, radio, cloud upload and the additional sensors, or has it been superseded?

- Is the relay intended to switch anything heavier than the sensor rail, such as a heater, fan or radio module? If so, the service life figures in section 4.3 must be recalculated.

- Does the clock module have a healthy backup cell fitted?

- Is gust reporting required for the intended data consumers?

- Was there a specific reason relays were selected rather than solid state load switches? For a load of a few milliamps, a high side switch would avoid the coil current and the wear entirely.

# 8. Configuration reference

All tunable values are grouped in a single configuration block at the top of the file. Nothing below that block should need editing to change station behaviour.

| Constant | Default | Purpose |
| --- | --- | --- |
| DEFAULT_SAMPLE_INTERVAL_S | 300 | Sampling cadence in seconds |
| MIN_SAMPLE_INTERVAL_S | 5 | Lower bound accepted by the console |
| MAX_SAMPLE_INTERVAL_S | 3600 | Upper bound, set by the 16 bit counter |
| SENSOR_WARMUP_MS | 2200 | Rail on to first valid reading |
| MIN_GATING_INTERVAL_S | 30 | Below this the rail stays energised |
| GUST_TRACKING_ENABLED | true | Enables three second gust sub sampling |
| GUST_SAMPLE_MS | 3000 | Gust window, standard meteorological value |
| RAIN_POLL_MS | 20 | Debounce polling cadence |
| RAIN_DEBOUNCE_MS | 100 | Minimum spacing between accepted tips |
| MM_PER_TIP | 0.2 | Rain gauge bucket volume in millimetres |
| WIND_MPH_PER_HZ | 1.5 | Anemometer calibration. See section 7.1. |
| CPU_FREQ_MHZ | 80 | Processor clock. Set to 240 for original speed. |

# 9. Limitations and verification status

This firmware is compile verified, not hardware verified. The build was performed against arduino-esp32 2.0.17 and ESP-IDF 4.4.7 and completes with zero warnings, using 23.6 percent of flash and 6.8 percent of available memory. The I2C libraries were inspected directly to confirm that repeated initialisation does not leak.

The following require confirmation on the bench:

- Pulse counter behaviour with the actual reed switches fitted, particularly whether the 100 millisecond rain debounce is correctly tuned for these specific gauges.

- Whether the 2200 millisecond warmup is sufficient for every device on the rail in the deployed enclosure and at low temperature.

- Relay switching behaviour, including whether energising the rail causes a supply dip large enough to disturb the processor. If unexplained restarts are observed, inrush current on the rail is the first thing to investigate.

- Absolute power consumption figures, which depend entirely on the relay fitted.

One further note on the toolchain. This build uses the current pulse counter interface for ESP-IDF 4.x. If the project is later moved to arduino-esp32 version 3.x, which is based on ESP-IDF 5.x, that interface still compiles but is formally deprecated in favour of a newer one. All calls are contained within a single function, so the migration would be localised.